SIMATS ENGINEERING

Department of Computer Science and Engineering

ASSESSMENT TOOL 1 – CONSTRAINT-BASED PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 – Precision (BL4) | Assessment: | Constraint Based Problem Solving |
| Weightage: | 40% | Total Marks: | 100 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: |  | Reg. No.: |  |
| Section / Batch: |  | Date: |  |

Instructions to Students - Identify the relevant concepts of I/O interfaces, interrupts, DMA, bus arbitration, and high-speed communication protocols. - Analyze the I/O requirements considering CPU utilization, latency, throughput, bandwidth, and device priorities. - Apply suitable I/O techniques such as DMA, vectored interrupts, memory-mapped I/O, nested interrupts, and bus arbitration. - Compute performance measures such as data transfer rate, interrupt latency, CPU utilization, throughput, and transfer time. - Interpret the results by evaluating CPU efficiency, communication performance, interrupt response, and suitability of the proposed I/O architecture.

QUESTIONS

1. Construct an I/O communication mechanism for a real-time embedded system with the following constraints:

- CPU utilization must be below 15%

- Multiple sensors generate interrupts every 1 ms

- Use vectored interrupt handling

- Select between memory-mapped and I/O-mapped I/O for optimal latency

2.Design an I/O subsystem under these constraints:

- High-speed storage requires throughput > 3 GB/s

- Must use NVMe over PCIe

- CPU should not handle bulk data transfer

- Integrate DMA controller with interrupt notification

3.Construct a multi-device communication architecture with constraints:

- 6 peripherals using USB, 2 high-speed devices using Thunderbolt

- Avoid interrupt starvation

- Use hardware and software interrupts

- Ensure fair bus arbitration

4.Design a data acquisition system with constraints:

- Continuous data stream at 500 MB/s

- Minimal CPU intervention required

- Must compare DMA transfer vs interrupt-driven I/O

- Use memory-mapped I/O for device registers

5.Construct an interrupt handling mechanism with constraints:

- Support nested interrupts

- Maximum interrupt latency < 10 µs

- Use vectored interrupts for high-priority devices

- Use software interrupts for background tasks

# Code of Conduct

I __________________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the Student: _________________________

RUBRICS FOR CALCULATION

| Criteria | Weightage(%) | Level 4 – Exemplary (5 Marks) | Level 3 – Proficient (4 Marks) | Level 2 – Developing (2–3 Marks) | Level 1 – Beginning (0–1 Mark) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 25% | Clearly defines the problem and identifies all relevant constraints accurately | Identifies most constraints with minor gaps | Partial understanding; misses key constraints | Fails to identify constraints correctly |
| Constraint Analysis | 25% | Analyzes constraints effectively and prioritizes them logically | Provides reasonable analysis with some prioritization | Limited analysis; weak prioritization | No meaningful analysis or prioritization |
| Solution Development | 25% | Develops optimal and feasible solutions satisfying all constraints | Develops workable solutions with minor limitations | Solutions partially satisfy constraints | Solutions do not meet constraints |
| Application of Concepts | 15% | Effectively applies appropriate concepts, methods, or tools | Applies concepts with minor errors | Limited application; requires guidance | Unable to apply relevant concepts |
| Justification | 10% | Provides strong justification for decisions with logical reasoning | Provides justification with some clarity | Weak or unclear justification | No justification provided |

Marks Evaluation:

| Q. No | Problem Understanding (25) |  | Constraint Analysis (25) |  | Solution Development (25) | Application of Concepts (15) | Justification (10) | Total (out of 100) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 2 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 3 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 4 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 5 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
|  | / 25 |  | /25 |  | / 25 | / 15 | / 10 | / 100 |

Course Coordinator  	 	              	 	 	Faculty Incharge

SET 1

CO 5 ASSESSMENT TOOL 1 – CONSTRAINT-BASED PROBLEM SOLVING

# Question 1 – I/O Mechanism for a Real-Time Embedded System

## Given Constraints

- CPU utilization must be below 15%

- Multiple sensors generate interrupts every 1 ms

- Vectored interrupt handling must be used

- Choose memory-mapped or I/O-mapped I/O for optimal latency

## 1. Problem Understanding

A real-time embedded controller must service several sensors that each raise an interrupt once every 1 ms (rate = 1000 Hz per sensor). The design must keep the processor mostly free for the main control loop (utilization < 15%), guarantee that the correct handler runs immediately when an interrupt fires (vectored interrupts), and pick an I/O addressing style that minimises access latency to the sensor registers.

## 2. Constraint Analysis & Selected Technique

Interrupt handling style: Vectored interrupts are specified directly. Each sensor is wired to a distinct interrupt line/ID, and the interrupt controller (e.g., an ARM Cortex-M NVIC style controller) holds a vector table so the CPU jumps straight to the correct Interrupt Service Routine (ISR) without a software polling loop. This removes the overhead of a generic handler that has to poll status registers to find which device interrupted, which is essential for meeting a hard 1 ms deadline for several sensors at once.

Addressing style: Memory-mapped I/O is selected over isolated (I/O-mapped) I/O for this system.

- Sensor registers appear in the normal address space, so they can be read/written with ordinary load/store instructions – no special IN/OUT instructions or extra decode cycles are needed, shortening ISR entry time.

- Compilers can use pointer/struct access directly to registers, which shortens and simplifies ISR code – important when the ISR budget is small.

- Almost all embedded microcontrollers (ARM Cortex-M, RISC-V based MCUs) already integrate peripherals into a single unified memory map, so vectored-interrupt hardware and memory-mapped registers work together natively.

- The only defence needed is to mark the peripheral address region as "Device/non-cacheable" so the CPU does not cache stale sensor readings.

## 3. Performance Computation

Assume N = 4 sensors, each interrupting every 1 ms (1000 interrupts/sec each), so the combined interrupt rate is:

Total interrupt rate = 4 sensors × 1000 Hz = 4000 interrupts/sec

To keep CPU utilization below 15%, the total time spent inside all ISRs per second must satisfy:

Utilization = (Interrupt rate × ISR service time) ≤ 0.15

ISR service time ≤ 0.15 / 4000 = 37.5 µs per interrupt (averaged across all sensors)

Because memory-mapped access removes special-instruction overhead and vectored dispatch removes the polling loop, a typical ISR (read sensor register, timestamp, buffer the sample, clear flag, return) can comfortably execute in 5–10 µs on a 50–100 MHz microcontroller, well inside the 37.5 µs budget, leaving CPU utilization for interrupt handling around 2–3%.

## 4. Interpretation

The architecture (memory-mapped registers + vectored, per-sensor interrupts) satisfies every constraint: the < 15% CPU-utilization target is met with margin, the 1 ms sensor cadence is served on time because vectoring removes dispatch delay, and memory-mapped addressing gives the fastest possible register access with the simplest ISR code — making it the correct choice over I/O-mapped I/O for this latency-sensitive, multi-sensor design.

# Question 2 – I/O Subsystem for High-Speed NVMe Storage

## Given Constraints

- High-speed storage requires throughput > 3 GB/s

- Must use NVMe over PCIe

- CPU should not handle bulk data transfer

- Integrate a DMA controller with interrupt notification

## 1. Problem Understanding

The storage subsystem must sustain more than 3 GB/s of throughput while keeping the CPU almost entirely out of the data path, since bulk copying by the processor would both be too slow and would defeat the CPU-utilization goals of the overall system.

## 2. Constraint Analysis & Selected Technique

Bus / protocol: NVMe (Non-Volatile Memory Express) running over a PCIe link is used because NVMe is designed from the ground up for low-latency, high-queue-depth SSD access, replacing the older AHCI/SATA model that was designed for spinning disks.

Bus bandwidth check: A PCIe Gen4 x4 link runs each lane at 16 GT/s with 128b/130b encoding, giving about 1.97 GB/s per lane.

Link throughput = 4 lanes × 1.97 GB/s ≈ 7.88 GB/s

This comfortably clears the > 3 GB/s requirement even after protocol overhead, so a PCIe Gen4 x4 (or even Gen3 x4, ≈ 3.94 GB/s) NVMe link is sufficient; Gen4 x4 is chosen for headroom.

Data movement: The NVMe controller's own DMA engine performs scatter–gather transfers directly between the SSD and system RAM using the NVMe submission/completion queue mechanism. The CPU only writes a command descriptor (address, length, opcode) into the submission queue — it never touches the bulk data itself.

Notification: The DMA engine raises a single MSI-X interrupt when a whole transfer (not each byte or word) completes, and the driver reads the completion queue entry. This is exactly the "DMA controller with interrupt notification" required by the constraint.

## 3. Performance Computation

Time to move a 1 MB block at the guaranteed throughput:

Transfer time = Data size / Throughput = 1 MB / 3 GB/s ≈ 341 µs

At the achieved PCIe Gen4 x4 link rate of 7.88 GB/s:

Transfer time = 1 MB / 7.88 GB/s ≈ 130 µs

If the DMA engine transfers in 128 KB bursts and interrupts once per burst, the interrupt rate at 3 GB/s sustained throughput is:

Interrupt rate = 3 GB/s ÷ 128 KB ≈ 23,400 interrupts/sec

— a manageable rate for a modern CPU (well under typical limits of a few hundred thousand interrupts/sec), confirming the CPU overhead stays low.

## 4. Interpretation

NVMe-over-PCIe Gen4 x4 exceeds the 3 GB/s target by more than 2×, the DMA engine keeps the CPU out of the bulk-transfer path entirely, and burst-level (rather than per-word) interrupts keep interrupt overhead small — together satisfying all four constraints while leaving the CPU free for other work.

# Question 3 – Multi-Device Communication Architecture (USB + Thunderbolt)

## Given Constraints

- 6 peripherals using USB, 2 high-speed devices using Thunderbolt

- Avoid interrupt starvation

- Use hardware and software interrupts

- Ensure fair bus arbitration

## 1. Problem Understanding

Eight devices of two very different speed classes (6 low/medium-speed USB peripherals and 2 very high-speed Thunderbolt devices) must share the host's interrupt and bus resources such that no device — especially a slow USB peripheral — is ever indefinitely denied service by the faster Thunderbolt traffic.

## 2. Constraint Analysis & Selected Technique

USB path (6 peripherals): USB does not use raw electrical interrupt lines from every device; instead the host controller polls each device's interrupt endpoint at a bounded, negotiated interval (typically 1–255 ms, e.g., 1 ms for HID devices, longer for slower peripherals). Because every device's poll interval is fixed at enumeration time and the host scheduler (EHCI/xHCI) reserves a guaranteed slot for it in each USB frame, no USB device can be starved indefinitely — each is guaranteed to be serviced within its own negotiated interval.

Thunderbolt path (2 devices): Thunderbolt tunnels PCIe and DisplayPort traffic and uses native hardware interrupts (MSI/MSI-X) straight to the CPU's interrupt controller for low latency, since Thunderbolt devices (e.g., an external GPU or high-speed capture device) need immediate servicing.

Two-level interrupt handling (hardware + software): Each hardware interrupt (from Thunderbolt devices or the USB host controller) is kept minimal — the top half (hardware ISR) only acknowledges the interrupt and schedules a deferred routine. The actual data processing (copying buffers, updating device state) is done in a software interrupt / deferred procedure (a "bottom half", e.g., a softirq or tasklet in an OS scheduler). This two-level split, drawn directly from the constraint ("hardware and software interrupts"), prevents any single device's hardware ISR from running long enough to block the others, which is the principal cause of interrupt starvation.

Bus arbitration: A weighted round-robin (WRR) arbitration scheme is used at the host controller: each of the 6 USB ports gets an equal round-robin share of USB frame time (fair arbitration among peripherals), while the 2 Thunderbolt devices are additionally granted reserved, isochronous-style bandwidth slots so their real-time streams are never blocked, without ever excluding the USB ports from their guaranteed slot. This combination — round-robin fairness for USB plus reserved slots for Thunderbolt — avoids starving either class of device.

## 3. Performance / Feasibility Check

If a 1 ms USB (micro)frame is divided among 6 peripherals in round robin, each is guaranteed at least 1/6 of a frame's bandwidth even under full Thunderbolt load, i.e. each USB peripheral gets a slot at least every 6 ms in the worst case — a bounded, provably non-starving wait, versus an unbounded wait under naive fixed-priority arbitration that always favours Thunderbolt.

## 4. Interpretation

Splitting interrupts into hardware (fast acknowledge) and software (deferred processing) tiers, combined with round-robin arbitration for the six USB peripherals and reserved bandwidth slots for the two Thunderbolt devices, gives every device a bounded worst-case wait time — meeting the "avoid interrupt starvation" and "fair bus arbitration" constraints simultaneously.

# Question 4 – Data Acquisition System (DMA vs Interrupt-Driven I/O)

## Given Constraints

- Continuous data stream at 500 MB/s

- Minimal CPU intervention required

- Must compare DMA transfer vs interrupt-driven I/O

- Use memory-mapped I/O for device registers

## 1. Problem Understanding

A data-acquisition front end (e.g., an ADC array or high-speed sensor bank) streams data continuously at 500 MB/s into system memory. The design must decide how each data unit reaches memory with minimal CPU load, and must use memory-mapped registers to configure/monitor the acquisition hardware.

## 2. Constraint Analysis — DMA vs Interrupt-Driven I/O

Interrupt-driven I/O (rejected): if the CPU is interrupted for every data unit, assume a 4-byte register read per interrupt:

Interrupt rate = 500 MB/s ÷ 4 bytes = 125,000,000 interrupts/sec

This is orders of magnitude beyond what any general-purpose CPU can service (practical ceilings are on the order of 10^5–10^6 interrupts/sec once context-save/restore overhead is included), so pure interrupt-driven transfer is infeasible at this data rate.

DMA transfer (selected): the DMA controller moves whole blocks from the acquisition hardware into memory autonomously and interrupts the CPU only once per block. Using, e.g., a 64 KB DMA block size:

Interrupt rate = 500 MB/s ÷ 64 KB ≈ 7,629 interrupts/sec

This is easily within normal CPU capability, so DMA is the technique used, matching the "minimal CPU intervention" constraint directly.

Register access: The DMA controller's configuration registers (source address = ADC FIFO, destination address = ring buffer in RAM, block size, start/status bits) are exposed as memory-mapped I/O, so the driver sets up a transfer with ordinary stores to fixed addresses and reads a status/interrupt register the same way — satisfying the last constraint directly.

## 3. Performance Computation

Time to fill one 64 KB DMA block at 500 MB/s:

Block time = 64 KB / 500 MB/s ≈ 131 µs

CPU work per block is limited to servicing one short DMA-completion ISR (a few microseconds to swap buffers / re-arm the DMA), so CPU utilization for the whole 500 MB/s stream stays in the low single digits, versus being completely saturated (>100× over capacity) under interrupt-driven transfer.

## 4. Interpretation

The comparison shows interrupt-driven I/O cannot physically sustain 500 MB/s because the required interrupt rate exceeds CPU capability by roughly four orders of magnitude, whereas block-based DMA reduces the interrupt rate to a few thousand per second — a genuinely minimal CPU load. Memory-mapped device registers keep the control-path code simple. DMA is therefore the correct and necessary technique for this data-acquisition system.

# Question 5 – Interrupt Handling Mechanism (Nested, <10 µs Latency)

## Given Constraints

- Support nested interrupts

- Maximum interrupt latency < 10 µs

- Use vectored interrupts for high-priority devices

- Use software interrupts for background tasks

## 1. Problem Understanding

The system must react to high-priority hardware events within 10 µs, must allow a higher-priority interrupt to pre-empt (nest inside) an already-running lower-priority ISR, and must separate genuinely time-critical work from background/housekeeping work.

## 2. Constraint Analysis & Selected Technique

Nested, priority-based controller: A priority-based Nested Vectored Interrupt Controller (NVIC-style, as in ARM Cortex-M or an equivalent priority-encoded PIC) is used. Every interrupt source is assigned a priority level; if a higher-priority interrupt arrives while a lower-priority ISR is executing, the controller automatically pre-empts the running ISR, stacks its context, and jumps to the higher-priority handler — this is exactly "nested interrupts."

Vectored dispatch for high-priority devices: each high-priority hardware device has its own entry in the vector table, so the CPU jumps directly to its ISR (no software polling/decoding chain), which is essential to keep worst-case latency low.

Software interrupts for background tasks: non-critical, deferrable work (logging, statistics, UI refresh, cleanup) is triggered via a software interrupt / trap instruction (e.g., SVC/SWI) invoked by program code rather than an external signal, and is assigned the lowest priority so it can never block or delay a high-priority hardware interrupt, and is itself pre-emptable by any hardware interrupt.

## 3. Latency Budget Computation

Assume a 100 MHz core clock (10 ns per cycle). The 10 µs budget corresponds to:

Cycle budget = 10 µs / 10 ns = 1000 cycles

A vectored controller typically needs: interrupt-controller response + vector fetch ≈ 2–5 cycles, automatic context (register) save ≈ 12–20 cycles (hardware-assisted, as in Cortex-M), and pipeline flush ≈ a few cycles — together on the order of 20–30 cycles (0.2–0.3 µs at 100 MHz).

Latency used = ~30 cycles × 10 ns ≈ 0.3 µs, leaving ≈ 9.7 µs of the 10 µs budget for the actual ISR body — a wide safety margin even after accounting for a possible one-level nesting pre-emption overhead (another ~20–30 cycles).

## 4. Interpretation

A priority-encoded, nested, vectored interrupt controller keeps hardware dispatch overhead to well under 1 µs, comfortably inside the 10 µs ceiling even with nesting; reserving software interrupts strictly for background tasks at the lowest priority guarantees they can never inflate the response time of a high-priority hardware event. All four constraints — nesting, <10 µs latency, vectoring for high-priority devices, and software interrupts for background work — are satisfied together.