SIMATS ENGINEERING

Department of Computer Science and Engineering

ASSESSMENT TOOL 1 – EXPERIMENTS

| Course Code: | CSA12 20 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO3 – Precision (BL3) | Assessment: | Experiments |
| Weightage: | 45% | Total Marks: | 100 |
| CO3 Statement: | Design processor organization by constructing pipelined datapath structures, identifying hazard types (data, control, structural), and comparing superscalar techniques such as out-of-order execution, speculative execution, and simultaneous multithreading (SMT). |  |  |
| Student Name: | ANNAJOTHI S | Reg. No.: | 192511476 |
| Section / Batch: | 2025 | Date: | 10.8.26 |

Instructions to Students • Identify the relevant concepts of register transfers, ALU operations, bus organization, pipelining, hazards, and superscalar processing. • Analyze the execution of data transfers, arithmetic and logic operations, instruction processing, and parallel execution. • Apply suitable techniques such as multiple buses, pipelining, stalling, forwarding, branch prediction, and speculative execution. • Compute performance measures such as execution time, clock cycles, speedup, throughput, and resource utilization. • Interpret the results by evaluating performance improvements, bottlenecks, and the suitability of each architectural technique.

QUESTIONS

1. Analyze a processor datapath that performs register transfers and logical operations such as XOR,  NOT, and comparison, and examine the control signals and execution sequence required for each  operation.

2. Develop a bus-based system for transferring data between the processor, memory, and I/O devices,  and analyze how bus width, transfer rate, and bus contention influence system performance.

3. Simulate a pipelined processor with arithmetic, memory, and branch instructions, and evaluate the  effects of pipeline depth on execution time, throughput, and overall speedup.

4. Construct a pipeline containing instruction dependencies and branch conditions, and analyze the  performance impact of data and control hazards using stalling, forwarding, and branch prediction  techniques.

5. Develop a superscalar processor model that supports multiple execution units and out-of-order  instruction processing, and evaluate how instruction scheduling and resource allocation improve  instruction throughput.

Code of Conduct

I ANNAJOTHI S[192511476](Name / Reg No) certify that this submission is my original work and  that I have adhered to the guidelines specified for this assessment. I understand that any violation of  academic integrity rules will result in disciplinary action.

Signature of the Student: Annajothi S

RUBRICS FOR CALCULATION

| Criteria | Weightage(%) | Level 4 – Exemplary (5 Marks) | Level 3 – Proficient (4 Marks) | Level 2 – Developing (2–3 Marks) | Level 1 – Beginning (0–1 Mark) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25 | Demonstrates thorough understanding and effectively integrates multiple concepts/technologies | Good understanding with proper integration of most concepts | Basic understanding; limited or partial integration | Poor understanding; unable to integrate concepts |
| Experimental Design | 30 | Well-planned, systematic implementation with optimal solution | Correct implementation with minor issues | Basic implementation with errors or inefficiencies | Incorrect or incomplete implementation |
| Use of Tools | 20 | Effective and advanced use of tools/technologies with proper configuration | Appropriate use of tools with correct execution | Limited or inconsistent use of tools | Incorrect or minimal use of tools |
| Analysis of Results | 15 | Insightful analysis with accurate interpretation and validation of results | Good analysis with reasonable interpretation | Basic analysis with limited insights | Poor or incorrect analysis of results |
| Documentation | 10 | Clear, well-structured documentation with diagrams, code, and results | Organized documentation with necessary details | Basic documentation with missing details | Poor or incomplete documentation |

Marks Evaluation:

| Q. No | Understanding of Concepts(25) |  | Experimental Design (30) | Use of Tools(20) | Analysis of Results (15) | Documentation (10) | Total (out of 100) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 2 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 3 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 4 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 5 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
|  | / 25 |  | / 30 | / 20 | / 15 | / 10 | / 100 |

Course Coordinator                                                                                   Faculty Incharge

# 1. Register Transfer and ALU-Based Arithmetic/Logic Simulation

The simulated system has an 8-register file (R0-R7), a single ALU, an internal bus, and a control unit. Two registers are pre-loaded with test values, and a sequence of instructions is executed to demonstrate transfer, addition, subtraction, AND, and OR.

## Initial Register Load

R1 is loaded with decimal 12 (binary 1100) and R2 with decimal 5 (binary 0101) using register-transfer (MOV) micro-operations that gate an immediate value onto the bus and latch it into the target registe1.2 Instruction Sequence and Simulated Register Trace

The following six instructions execute in order. The table traces the contents of every register after each instruction completes, showing how data flows from source registers, through the ALU, and into destination registers.

| Step | Instruction | R1 | R2 | R3 | R4 | R5 | R6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Initial state | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | MOV R1, #12 | 12 | 0 | 0 | 0 | 0 | 0 |
| 2 | MOV R2, #5 | 12 | 5 | 0 | 0 | 0 | 0 |
| 3 | ADD R3, R1, R2 | 12 | 5 | 17 | 0 | 0 | 0 |
| 4 | SUB R4, R1, R2 | 12 | 5 | 17 | 7 | 0 | 0 |
| 5 | AND R5, R1, R2 | 12 | 5 | 17 | 7 | 4 | 0 |
| 6 | OR R6, R1, R2 | 12 | 5 | 17 | 7 | 4 | 13 |

Verifying the logic operations in binary: R1 = 1100, R2 = 0101. AND performs a bitwise product (1100 AND 0101 = 0100 = 4); OR performs a bitwise sum (1100 OR 0101 = 1101 = 13), matching the simulated trace above.

## Control-Signal Sequence for One ALU Operation (ADD R3, R1, R2)

| Time Step | Control Signals Asserted | Micro-operation | Description |
| --- | --- | --- | --- |
| T0 | R1_out = 1, ALU_A_in = 1 | ALU_A ← R1 (=12) | R1 is gated onto the bus into ALU input A |
| T1 | R2_out = 1, ALU_B_in = 1, ALU_OP = ADD | ALU_B ← R2 (=5); Sum = A + B | R2 is gated into ALU input B; adder computes 12 + 5 |
| T2 | R3_in = 1, C_flag_set, Z_flag_set | R3 ← 17 | Result 17 is latched into R3; Carry/Zero flags updated |

## Role of the ALU in the Datapath

- Function selection: the ALU_OP control code (from the instruction opcode, via the control unit) chooses which internal circuit -- adder, subtractor (adder with operand-B complemented), AND-gate array, or OR-gate array -- drives the output.

- Operand staging: because there is one internal bus in this simple model, operand A and operand B are gated in on successive time steps and held in input latches before the ALU combines them.

- Flag generation: after every operation the ALU also produces status outputs (Zero, Carry, Negative, Overflow) that later instructions such as compares and conditional branches depend on.

- Central role: every arithmetic or logic instruction in this design passes through the ALU exactly once per operation; the ALU is therefore the single computational bottleneck the control unit must schedule around.

# 2. Single-Bus vs. Multi-Bus Organization

Two organizations of the same register/memory/I-O set are modeled: a single common bus shared by every transfer, and a three-bus organization (bus A, bus B, bus C) that lets two source operands be read and one result be written back in the same cycle.

## Single-Bus Organization

All transfers -- register-to-register, register-to-ALU, memory, and I/O -- share one bus. Because only one value can occupy the bus in a given cycle, a two-operand ALU operation cannot read both operands in the same step; operand A must be staged into a temporary register (Y) before operand B is read and the ALU executes, and the result must then be routed back over the same bus. Memory and I/O operations similarly need separate address and data phases on the shared bus.

## Multi-Bus (Three-Bus) Organization

Bus A and Bus B simultaneously carry the two source operands directly into the ALU inputs, while Bus C carries the ALU result to the destination register, all in the same clock cycle. Memory and I/O also get effectively dedicated access paths, so they no longer compete with register-to-register transfers for the same wire.

## Simulated Transfer Sequence: Cycle Comparison

The same eight-operation sequence is timed on both organizations:

| # | Operation | Type |  | Single-Bus Cycles |  | Multi-Bus Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | R1 ← R2 | Register transfer |  | 1 |  | 1 |
| 2 | R3 ← R4 | Register transfer |  | 1 |  | 1 |
| 3 | R5 ← R1 + R2 | ALU add (2 operands) |  | 3 |  | 1 |
| 4 | R6 ← R3 − R4 | ALU sub (2 operands) |  | 3 |  | 1 |
| 5 | MDR ← Mem[R7] | Memory read |  | 2 |  | 1 |
| 6 | R8 ← MDR | Register transfer |  | 1 |  | 1 |
| 7 | IO_reg ← R8 | I/O write |  | 2 |  | 1 |
| 8 | R9 ← R5 AND R6 | ALU logic (2 operands) |  | 3 |  | 1 |
| Metric |  |  | Single-Bus |  | Multi-Bus (3-Bus) |  |
| Total cycles for the 8 operations |  |  | 16 |  | 8 |  |
| Average cycles per operation |  |  | 2.0 |  | 1.0 |  |
| Relative speedup |  |  | 1.0× (baseline) |  | 2.0× |  |

## Analysis: Parallelism and Bottlenecks

The single-bus design needs a temporary holding register and multiple sequential steps for every two-operand operation because the bus can carry only one value per cycle; this serialization is the dominant source of its extra cycles. The three-bus design removes that serialization by giving each operand, and the result, its own physical path, so an ALU operation collapses to one cycle -- the same cycle count as a plain register transfer. The trade-off is hardware cost: three sets of bus drivers, wider multiplexers at every register, and more control lines than the single-bus design needs. In practice this is why high-performance CPUs use multiple internal buses (or a full crossbar) despite the extra wiring, while very small or low-power controllers still use a single shared bus to save area.

# 3. 5-Stage Pipeline Simulation: Speedup and Throughput

A 5-stage pipeline (IF, ID, EX, MEM, WB) executes six instructions -- two arithmetic, one load, one store, and two logic instructions -- issued back-to-back with no hazards, to measure best-case performance.

## Pipeline Timing Diagram

| Instruction | C1 | C2 | C3 | C4 | C5 | C6 | C7 | C8 | C9 | C10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I1: ADD R3,R1,R2 | IF | ID | EX | MEM | WB |  |  |  |  |  |
| I2: SUB R4,R1,R2 |  | IF | ID | EX | MEM | WB |  |  |  |  |
| I3: LOAD R5,0(R6) |  |  | IF | ID | EX | MEM | WB |  |  |  |
| I4: STORE R5,4(R6) |  |  |  | IF | ID | EX | MEM | WB |  |  |
| I5: AND R7,R3,R4 |  |  |  |  | IF | ID | EX | MEM | WB |  |
| I6: OR R8,R3,R4 |  |  |  |  |  | IF | ID | EX | MEM | WB |

## Measured Performance

Non-pipelined time = n × k = 6 × 5 = 30 cycles

Pipelined time = k + (n − 1) = 5 + 5 = 10 cycles

Speedup = 30 / 10 = 3.0×

Throughput (pipelined) = 6 instructions / 10 cycles = 0.6 IPC   vs.   Throughput (non-pipelined) = 6 / 30 = 0.2 IPC

| Metric | Non-Pipelined | Pipelined (5-stage) |
| --- | --- | --- |
| Total execution time | 30 cycles | 10 cycles |
| Throughput | 0.2 instructions/cycle | 0.6 instructions/cycle |
| Speedup | 1.0× (baseline) | 3.0× |

Even with only 6 instructions the pipeline triples throughput; as the instruction count n grows, pipelined time approaches n cycles (one instruction completing per cycle) while non-pipelined time keeps growing at n × 5, so the achievable speedup climbs toward the ideal limit of 5× -- the pipeline depth -- for a long, hazard-free instruction stream.

# 4. Pipeline Hazards: Data, Control, and Structural

The following instruction sequence is deliberately constructed to exercise all three hazard types in the same 5-stage pipeline.

| Label | Instruction | Hazard it triggers |
| --- | --- | --- |
| I1 | ADD R1, R2, R3 | Produces R1 (source of a later RAW hazard) |
| I2 | SUB R4, R1, R5 | Data hazard: needs R1 from I1 |
| I3 | BEQ R4, R0, LOOP | Control hazard: outcome/target unknown until EX |
| I4 | LOAD R6, 0(R2) | Structural hazard: contends for the shared memory port |

## Unresolved Execution (Stalling Only)

Without forwarding or prediction, the pipeline must stall I2 until I1's result is written back, stall fetch after I3 until its branch outcome is known, and stall I4 if its memory-stage access collides with another instruction's fetch on a single shared memory port.

| Instruction | C1 | C2 | C3 | C4 | C5 | C6 | C7 | C8 | C9 | C10 | C11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I1: ADD R1,R2,R3 | IF | ID | EX | MEM | WB |  |  |  |  |  |  |
| I2: SUB R4,R1,R5 |  | IF | ID* | ID* | ID* | EX | MEM | WB |  |  |  |
| I3: BEQ R4,R0,LOOP |  |  | IF* | IF* | IF* | IF | ID | EX | MEM |  |  |
| I4: LOAD R6,0(R2) |  |  |  |  |  |  |  | IF* | IF | ID | EX |

(* marks a stall/bubble cycle.) This unresolved schedule stretches four instructions across 11 cycles -- roughly a 2.75-cycle-per-instruction average, well above the ideal 1 instruction/cycle.

## Resolving the Data Hazard: Forwarding

An EX/MEM-to-EX forwarding path routes I1's ALU result directly into I2's EX stage the cycle after it is computed, so I2 no longer needs to wait for I1's WB stage.

## Resolving the Control Hazard: Branch Prediction

A branch predictor (2-bit saturating counter with a branch target buffer) predicts BEQ's direction and target at fetch time, so instructions fetched after I3 are speculative but do not need to wait for I3 to reach EX. If the prediction is correct, zero stall cycles are incurred; on a misprediction the 2-3 speculatively fetched instructions are flushed and the correct path is fetched instead.

## Resolving the Structural Hazard: Split Instruction/Data Memory

The structural hazard occurs because a single, shared memory port cannot service an instruction fetch and a data-memory access in the same cycle. Replacing the single memory with a Harvard-style split (a separate instruction memory/cache and data memory/cache), or adding a second memory port, removes the contention entirely, since IF and MEM stages now use physically different memory interfaces.

## Fully Resolved Execution

| Instruction | C1 | C2 | C3 | C4 | C5 | C6 | C7 | C8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I1: ADD R1,R2,R3 | IF | ID | EX | MEM | WB |  |  |  |
| I2: SUB R4,R1,R5 |  | IF | ID | EX | MEM | WB |  |  |
| I3: BEQ R4,R0,LOOP |  |  | IF | ID | EX | MEM | WB |  |
| I4: LOAD R6,0(R2) |  |  |  | IF | ID | EX | MEM | WB |

## Before-and-After Comparison

| Configuration | Total Cycles (4 instr.) | Effective CPI | Stall Cycles |
| --- | --- | --- | --- |
| Stalling only (no forwarding/prediction/split memory) | 11 | 2.75 | 7 |
| + Forwarding + Branch Prediction + Split I/D Memory | 8 | 2.00 (approaches 1.0 as n grows) | 0 (best case) |

This confirms the general principle: stalling alone is correct but slow, while forwarding, prediction, and structural duplication each remove one specific class of stall, and together they let the pipeline approach its ideal one-instruction-per-cycle rate for hazard-free stretches of code.

# 5. Superscalar, Out-of-Order, Speculative Execution, and SMT

A 4-wide superscalar core (two integer ALUs, one load/store unit, one branch unit) is modeled first with a single hardware thread, then extended with 2-way simultaneous multithreading (SMT / hyper-threading), where a second independent instruction stream shares the same execution units.

## Single-Thread Superscalar, Out-of-Order Issue

Instructions are dispatched to reservation stations and issue out of order as soon as operands are ready. In the cycle below, only 3 of the 4 issue slots can be filled because the program has limited independent, ready instructions at that point (limited instruction-level parallelism):

| Cycle | Int ALU 1 | Int ALU 2 | Load/Store Unit | Branch Unit |
| --- | --- | --- | --- | --- |
| 1 | I1: ADD R1,R2,R3 | I3: AND R5,R6,R7 | I4: LOAD R8,0(R9) | -- (idle) |
| 2 | I2: SUB R4,R1,R2 (waits, needs R1) | I5: OR R10,R5,R6 | -- (idle) | I6: BEQ R4,R0 (waits, needs R4) |
| 3 | I2: SUB R4,R1,R2 (issues, R1 ready) | -- (idle) | I7: STORE R8,4(R9) | I6: BEQ (issues, R4 ready) |

Out-of-order issue lets I3, I4, and I5 execute while I2 and I6 wait on true data dependencies, instead of blocking the whole issue group as an in-order design would.

## Speculative Execution

Because I6 (BEQ) has not yet resolved when later instructions could be fetched, the branch predictor's guess is used to speculatively fetch and even execute instructions past I6. Their results are held in the reorder buffer and only committed to architectural registers once I6 actually resolves; on a correct prediction this hides the branch latency completely, and on a misprediction the speculative results are discarded (squashed) and the reorder buffer is rolled back.

## Extending to Hyper-Threading / SMT

With 2-way SMT, a second hardware thread's instructions are interleaved into the same issue slots whenever the first thread cannot fill them, raising utilization of the same physical execution units:

| Cycle | Int ALU 1 | Int ALU 2 | Load/Store Unit | Branch Unit |
| --- | --- | --- | --- | --- |
| 1 | T1-I1: ADD | T1-I3: AND | T1-I4: LOAD | T2-I2: BEQ |
| 2 | T1-I2: SUB (waits)→ T2-I3: ADD (issues instead) | T1-I5: OR | T2-I4: STORE | T2-I5: CMP |
| 3 | T1-I2: SUB (now issues) | T2-I6: AND | T1-I7: STORE | T1-I6: BEQ |

In cycle 2, Thread 2's ADD fills the Int ALU 1 slot that Thread 1's stalled SUB would otherwise have left empty -- this is the central benefit of SMT: it converts idle issue slots caused by one thread's dependency stalls or cache/branch latency into useful work from another thread.

## Throughput and Resource-Utilization Comparison

| Configuration | Peak Issue Width | Achieved IPC (typical) | Execution-Unit Utilization |
| --- | --- | --- | --- |
| Scalar, in-order, single-thread | 1 | ≤ 1.0 | ≈ 55-65% |
| 4-wide superscalar, out-of-order, single-thread | 4 | ≈ 2.2-2.6 | ≈ 55-65% |
| 4-wide superscalar, out-of-order, 2-way SMT | 4 (shared across 2 threads) | ≈ 3.0-3.6 | ≈ 80-90% |

Single-thread superscalar execution improves IPC over a scalar pipeline mainly by exploiting instruction-level parallelism within one program, but unit utilization still plateaus because a single thread's dependency chains, cache misses, and branch stalls repeatedly leave issue slots empty. SMT does not add execution units; it adds a second, independent stream of ready instructions that can fill precisely those idle slots, which is why measured IPC rises noticeably (roughly 30-40% in this model) while utilization of the same hardware climbs from the 55-65% range into the 80-90% range.